Hammir deployment algorithm

The Hammir deployment calculation is performed as a deterministic post-processing model using the time-dependent lithium supply schedule derived from the ARC deployment cases.

First, the cumulative requirement of 90 at.% enriched lithium obtained from the ARC deployment analysis is converted into an equivalent 30 at.% enriched lithium supply by down-blending with natural lithium. The monthly increment of this converted Li-30 supply is accumulated into an available lithium inventory.

At each monthly time step, the following procedure is applied:

- The newly available Li-30 supply during the current month is added to the inventory.

- The operational lithium requirement of the previously deployed Hammir fleet is subtracted.

- The remaining inventory is used to determine the number of additional Hammir units that can be deployed.

- The startup lithium required for newly deployed Hammir units is deducted from the inventory.

- The remaining lithium inventory is carried forward to the next month.

The startup lithium requirement for one Hammir plant is:

Li30,startupHammir=40.2t/plant

The annual Li-6 consumption is reported as 38.5 kg/yr. Since Hammir uses lithium enriched to 30 at.% Li-6, the corresponding enriched lithium requirement becomes:

Li30,opHammir=38.50.30=128.3 kg/yr/plant=0.1283 t/yr/plant

or, equivalently on a monthly basis,

Li30,op,monthHammir=0.128312=0.0107 t/month/plant

The monthly lithium inventory is updated according to:

Im=Im-1+ΔLi30,supply(m)-NH(m-1)Li30,op,monthHammir

where the inventory after operational consumption is used to determine the deployability of additional Hammir units.

The cumulative Hammir fleet size is then updated as:

NH(m)=NH(m-1)+Nnew(m)

In the present model, the deployment of additional Hammir units is constrained not only by the startup lithium requirement but also by the ability to sustain future operational lithium demand.

Specifically, a newly deployed Hammir unit is accepted only if the available lithium inventory remains non-negative throughout all subsequent monthly time steps:

Ij≥0∀j≥m

where jdenotes future monthly time steps following deployment at month m.

This constraint prevents unrealistically optimistic deployment that could otherwise arise from considering only startup lithium availability. As a result, the calculated Hammir fleet represents the maximum deployable fleet that can be continuously sustained under the lithium supply scenario implied by the ARC deployment case.

Nomenclature

Im : Available inventory of 30 at.% enriched lithium at month m[t].

ΔLi30,supply(m) : Newly available 30 at.% enriched lithium supply added during month m[t/month]. This quantity is obtained from the ARC-derived 90 at.% enriched lithium supply after down-blending with natural lithium.

NH(m) : Total number of deployed Hammir plants at month m.

NH(m-1) : Number of operating Hammir plants from the previous month.

Li30,startupHammir : Startup lithium requirement per Hammir plant [t/plant].

Li30,op,monthHammir : Monthly operational lithium requirement per Hammir plant [t/month/plant].