Sustainable Lithium-Constrained Deployment Algorithm

The deployment calculation is performed as a deterministic post-processing model using the time-dependent lithium supply schedule obtained from the ARC deployment analysis. For the EU-DEMO case, the ARC-derived Li-90 supply is used directly, whereas for the Hammir case, the ARC-derived Li-90 supply is first converted into an equivalent Li-30 supply through down-blending with natural lithium while conserving the amount of Li-6. The subsequent deployment procedure is otherwise identical for both reactor concepts.

Monthly lithium inventory update

At each monthly time step, the following sequence is applied:

- The newly available enriched lithium supply is added to the inventory.

- The operational lithium consumption of the previously deployed reactor fleet is deducted.

- The remaining inventory is evaluated to determine whether additional reactors can be deployed.

- If deployment is accepted, the startup lithium requirement of the newly deployed reactors is deducted.

- The remaining inventory is carried forward to the next monthly time step.

Accordingly, the lithium inventory is updated as

Im=Im-1+Sm-Nm-1Dop-ΔNmDstartup
where

- Im: available enriched lithium inventory at month m,

- Sm: enriched lithium supplied during month m,

- Nm-1: number of reactors already deployed before month m,

- Dop: operational lithium requirement per reactor per month,

- Dstartup: startup lithium requirement per reactor,

- ΔNm: number of reactors newly deployed during month m.

The cumulative fleet size is updated by

Nm=Nm-1+ΔNm

Determination of newly deployed reactors

The value of ΔNmis not prescribed a priori but is determined iteratively at each monthly time step. Starting from zero additional deployments, reactors are accepted one by one until further deployment becomes infeasible.

For each candidate reactor, two conditions must be simultaneously satisfied:

- The lithium inventory after deducting the startup lithium requirement remains non-negative.

- The resulting fleet can be continuously sustained throughout all subsequent monthly time steps, such that the projected lithium inventory never becomes negative after accounting for future operational lithium consumption.

Therefore, the number of newly deployed reactors is determined as the maximum feasible value satisfying

ΔNm=max⁡n∈N  |  Ikn≥0,∀k>m,

where Ikndenotes the projected lithium inventory at future month kassuming that nadditional reactors are deployed during month m.

Consequently, deployment is constrained not only by the startup lithium requirement but also by the ability to sustain the future operational lithium demand of the entire deployed fleet. This forward feasibility check prevents unrealistically optimistic deployment that could arise from considering only startup lithium availability. As a result, the calculated reactor fleet represents the maximum continuously sustainable deployment under the prescribed lithium supply scenario rather than the maximum fleet achievable based solely on startup lithium availability.